- WM796

- OPTIMIZATION OF BEST MANAGEMENT PRACTICES

- FOR BEEF CATTLE RANCHING

- IN THE LAKE OKEECHOBEE BASIN

- Statistical Analysis

- (Nutrient Concentrations)

July 31, 2002

by

J.C. Capece, B. Houssard, J. Barkauskaite

- Southern DataStream, Inc.,  P.O. Box 1577, LaBelle FL 33975

http://www.SouthernDataStream.com

and

K.L.  Campbell, K.M. Portier

Department of Agricultural and Biological Engineering

Institute of Food and Agricultural Sciences

More information available at:

http://www.agen.ufl.edu/~maerc

http://www.SouthernDataStream.com/maerc

## Table of Contents

Table of Contents	2

Table of Figures	3

Table of Tables	4

I. Objectives	5

II. Project design	5

III. Stages of the data analysis	6

a) Distribution of the data	6

b) Outliers (Chauvenet’s criterion)	7

c) Means Test	8

d) ANOVA / GLM Tests	8

IV. Data analysis	9

a) Distribution of data	9

b) Duplicate tests	19

c) GLM tests	20

d) Concentration vs. Treatment	21

V. Tukey’s w test	28

1.	Objective	28

2.	Tukey’s w procedure on Minitab	28

3.	Tests	29

Conclusion	38

Appendix 1: GLM tests with Minitab (1999)	39

Appendix 2: GLM tests with Minitab (2000)	40

Appendix 3: GLM Test with Minitab (2001)	41

Appendix 4	42

Key words:

histograms, normal and lognormal distribution, Kolmogorov-Smirnov test, Chauvenet’s criterion for outliers, 2-sample T-tests, Anova, General Linear Model tests, Tukey’s w test

## Table of Figures

Figure 1.  Example of a (non-normal) distribution.	6

Figure 2.  Example of a normal distribution.	7

Figure 3.  Histogram of NH3 concentration for the summer plots.	9

Figure 4.  Graph showing the 68% confidence interval for NH3.	10

Figure 5.  Histogram of log concentration NH3 for summer plots.	11

Figure 6:  Histogram of log concentration NH3 for winter sites	11

Figure 7.  Graph showing the 68% confidence interval for NOx.	12

Figure 8.  Histogram of log concentration NOx for summer sites.	13

Figure 9.  Histogram of log concentration NOx for winter sites.	13

Figure 10.  Graph showing the 68% confidence interval for TKN.	14

Figure 11.  Histogram of log concentration TKN for summer sites	15

Figure 12.  Histogram of log concentration TKN for winter sites.	16

Figure 13.  Graph showing the 68% confidence interval for TP.	17

Figure 14.  Histogram of log concentration TP for summer sites.	18

Figure 15.  Histogram of log concentration TP for winter sites.	18

Figure 16.  TP concentration vs. treatment.	21

Figure 17.  TKN concentration vs. treatment.	22

Figure 18.  NOx concentration vs. treatment.	22

Figure 19.  NH3 concentration vs. treatment.	22

Figure 20.  NH3 concentration vs. treatment.	23

Figure 21.  NOx concentration vs. treatment.	24

Figure 22.  TKN concentration vs. treatment.	24

Figure 23.  TP concentration vs. treatment.	25

Figure 24.  NH3 concentration vs. treatment.	26

Figure 25.  NOx concentration vs. treatment.	26

Figure 26.  TKN concentration vs. treatment.	27

Figure 27.  TP concentration vs. treatment.	27

## Table of Tables

Table 1.  Design of the stocking rate treatments demonstration project.	5

Table 2:  Means and standard deviation for NH3	10

Table 3:  Frequency of log NH3 concentration	10

Table 4.  Means and standard deviation for NOx.	12

Table 5.  Frequency of log NOx concentration.	12

Table 6.  Means and standard deviation for TKN.	14

Table 7.  Frequency of log TKN concentration.	15

Table 8.  Means and standard deviation for TP	16

Table 9.  Frequency of log TP concentration.	17

Table 10.  P-value of the T-Tests for replicate sites (2001).	19

Table 11.  P-value of the T-Tests for replicate sites (1999).	19

Table 12.  P-value of the T-Tests for replicate sites (2000).	19

Table 13.  P-value of the GLM tests (2001).	20

Table 14.  P-value of the GLM tests (1999).	20

Table 15.  P-value of the GLM tests (2000).	21

Table 16.  Means of concentration for each treatment.	23

Table 17.  Means of concentration for each treatment.	25

Table 18.  Tukey's test - Example.	29

Table 19.  Tukey's test - Summer pasture - NH3 – 1999.	29

Table 20.  Tukey's test - Winter pasture – NH3 – 1999.	30

Table 21.  Tukey's test - Summer pasture - NOx – 1999.	30

Table 22.  Tukey's test - Winter pasture - NOx – 1999.	30

Table 23.  Tukey's test - Summer pasture - TKN – 1999.	31

Table 24.  Tukey's test - Winter pasture - TKN – 1999.	31

Table 25.  Tukey's test - Summer pasture - TP – 1999.	31

Table 26.  Tukey's test - Winter pasture - TP – 1999.	32

Table 27.  Tukey's test - Summer pasture - NH3 – 2000.	32

Table 28.  Tukey's test - Winter pasture - NH3 – 2000.	32

Table 29.  Tukey's test - Sumer pasture - NOx – 2000.	33

Table 30.  Tukey's test - Winter pasture - NOx – 2000.	33

Table 31.  Tukey's test - Summer pasture - TKN – 2000.	33

Table 32.  Tukey's test - Winter pasture - TKN – 2000.	34

Table 33.  Tukey's test - Summer pasture - TP – 2000.	34

Table 34.  Tukey's test - Winter pasture - TP – 2000.	34

Table 35.  Tukey's test - Summer pasture - NH3 – 2001.	35

Table 36.  Tukey's test - Winter pasture - NH3 – 2001.	35

Table 37.  Tukey's test - Summer pasture - NOx – 2001.	35

Table 38.  Tukey's test - Winter pasture - NOx – 2001.	36

Table 39.  Tukey's test - Summer pasture - TKN – 2001.	36

Table 40.  Tukey's test - Winter pasture - TKN – 2001.	36

Table 41.  Tukey's test - Summer pasture - TP – 2001.	37

Table 42.  Tukey's test - Winter pasture - TP – 2001.	37

## I. Objectives

The MAERC Buck Island Ranch project seeks to evaluate the effects of cattle stocking rate on water quality. After gathering all the water quality result files, Laurence Amilhat performed the statistical analysis for the data of the years 1998/1999, that is to say the two first years of the project. However, the data did not yield clear conclusions. This paper reports the study and the conclusions that result from the 2001 data. This report can be also reach on the Internet at:

http://www.intellitemps.net/houssard/data_analysis/data_analysis.htm

## II. Project design

The demonstration project seeks to evaluate four cattle stocking rates of cow-calf units on bahiagrass pastures sites. The infrastructure consists of multiple experimental pastures fenced and ditched separately from each other, and are instrumented, so that all surface water runoff can be captured and analyzed.  The design is described in Table 1.

Table 1.  Design of the stocking rate treatments demonstration project.

| Block | Plot ID | Treatment |  |  |
| --- | --- | --- | --- | --- |
|  |  | Description | Cow-Calf Units | Acres/Unit |
| Winter | W4 & W7 | Control | 0 | N/A |
|  | W1 & W6 | Low | 15 | 5.3 |
|  | W2 & W8 | Medium | 20 | 4 |
|  | W3 & W5 | High | 35 | 2.3 |
| Summer | S1 & S8 | Control | 0 | N/A |
|  | S4 & S6 | Low | 15 | 5.3 |
|  | S2 & S7 | Medium | 20 | 4 |
|  | S3 & S5 | High | 35 | 2.3 |

These grazing blocks reflect the two principal pasturing regimes of a typical central Florida ranch.  Each study animal was assigned to a stocking rate at the beginning of the study and remains at this same stocking rate for the life of the project.  The winter sites are located on area containing a mixture of mature grasses, along with some bahiagrass.  Winter pastures are for the grazing of cows immediately after calving and during breeding (dry season).  The summer sites are improved pasture with bahiagrass and used for the grazing of cow-calf pairs.  All the experimental pastures are similar in design and instrumentation.  A data logger installed at each flume triggers the auto sampler based upon the flow volume and hydrograph geometry.  The resulting water samples are shipped to the laboratory then analyzed for phosphorus, nitrate, ammonia and total nitrogen.  Also, grab samples are taken manually and analyzed for soluble reactive phosphorus.  The data logger records several different parameters including the water depth before (upstream) and after (downstream) the flume, the year, date, time, and sample number.  The combination of these measurements and the laboratory results are saved in a database and then analyzed to assess the effectiveness of the stocking rate treatments.

## III. Stages of the data analysis

### a) Distribution of the data

First, it is important to have an idea of the data distribution. Indeed, each statistical test assumes that the data are normally distributed. This assessment was accomplished by producing frequency histograms.

The histograms reflect the kind of distribution, so it is important not to neglect this stage. Indeed, the choice of the width of intervals, called bin range, has an important influence on the shape of the distribution. If too many intervals are chosen then the number of values in each interval is small and the histogram appears flat and featureless. At the other extreme, if the histogram consists of only two or three intervals, then all the values will lie in those intervals and the shape of the histogram reveals little. There are no hard and fast rules about choosing the width of intervals but a useful rule of thumb is to choose about N = √n intervals, where n is the number of values. Once N has been rounded to a whole number, the interval width, w, can be calculating using:

w = range / N = ( max value – min value ) / N

Figure 1.  Example of a (non-normal) distribution.

The histogram shown in Figure 1 does not exhibit a normal distribution. Water quality data typically manifest a lognormal distribution. Therefore a log transformation is likely to yield a normal frequency distribution, as it can be seen on Figure 2.

Figure 2.  Example of a normal distribution.

It is possible to determine statistically if a distribution corresponds to a normal distribution or not. A relevant test is the Kolmogorov-Smirnov test, which compare an empiric distribution (Fn*(x)) to a theoretical continuous one (F(x)).  We just compare the infinite norm of the 2 functions (Dn = | Fn* - F |∞ = Sup {|Fn*(x)-F(x)|, x € IR } ) to a tabulate value dn. If Dn < dn (α), we can conclude that the distribution is a normal one (with a error of α %)

### b) Outliers (Chauvenet’s criterion)

Identification of outliers is important as they may indicate a novel effect such as the existence of incorrect recording of the measured value. Trying to identify the cause of an outlier is very important.

One approach to deciding whether an outlier should be removed from a group of values is to begin by asking the question: Given the scatter displayed by the data as a whole, what is the probability of obtaining a value at least as far from the mean as the outlier?

If this probability is very low then we have good statistical grounds for omitting the outlier from any further analysis. To carry out this study, we must assume that the scatter data is well described by the normal distribution. For each value x, we calculate

z = (x - <x> ) / s								[Eq.1]

where <x> is the mean, s the standard deviation

Then, we find the probability that a value differs from the mean by at least z standard deviations ( 2*Norm(-∞ < Z < -abs(z) ) where Norm is the normal distribution (a statistic table or the direct Excel function Normdist can be used). Then, we multiply this probability by n, the number of values. It results the number of values Nv we expect to be at least as far from the mean as the outlier. If Nv < 0.5, we can reject this number of the study.

### c) Means Test

By analyzing the treatment replicate results among themselves we seek to verify whether they are valid replicates (means not significantly different).  One option in performing this verification is to run a paired test, but this test requires equal sample sizes and the assumption that our samples are dependent rather than independent measurements.  Since neither of these assumptions holds true in our case, we instead choose to run a two-sample T-test.  This allows us to assume independent samples and to test the difference between means calculated from unequal size samples.

### d) ANOVA / GLM Tests

By performing an analysis of the variance (ANOVA), we seek to assess the significance of main effects on the nutrient concentrations and runoff loads. There are two effects in the MAERC pasture project, so the two-way variance analysis should be used.  The two effects studied are the type of Block (winter or summer pasture) and the Treatment (cow-calf stocking rates). These test were previously carried out for the analysis of 1998/1999.

Unfortunately, ANOVA tests require equal subclass numbers, which is not verified for our current study. This difficulty was avoided for the 1998-1999 analyses by doing the test on the means only. But, taking into account the data variation would be more interesting. It is the reason why we chose to submit the data to general linear model (GLM) tests. To perform these tests, we had to use Minitab because MS Excel cannot do them.

It would be inappropriate to develop here the GLM [see http://www.statsoftinc.com/textbook/stglm.html as part of this report] but the presentation of the computerization may be relevant in this paper. To run the test, Minitab needs 3 columns of same length:

- A column which specifies each value

- A column which specifies the block corresponding to the data (summer/winter)

- A column which specifies the treatment corresponding to the data (control/low/medium/high)

The results appear in a table (similar to a ANOVA table), gathering for each test (block/treatment/interaction block-treatment):

- Df (degrees of freedom)

- Seq SS (sum of squares)

- Adj SS (adjusted sum of squares)

- Adj MS (=Adj SS/Df)

- F (F statistic)

- P (probability of means being as different as observed if the null hypothesis is true)

The most important of these 6 numbers is the P-value. This value allows determination if the null hypothesis (stating that there is no main effect) must be rejected or not. If P < 0.05 (5%), we can conclude that there is a statistically significant effect.

## IV. Data analysis

### a) Distribution of data

#### i) NH3

The histogram shown in Figure 3 represents the distribution of NH3 for the summer plots. To make it clearer, the boxes have been replaced by points linked each together by lines. This avoids having to produce 16 different histograms figures or depicture 16 different bar sets in the same figure.

Figure 3.  Histogram of NH3 concentration for the summer plots.

Table 2:  Means and standard deviation for NH3

|  |  | Control |  | Low |  | Medium |  | High |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |
| Summer | Mean | 0.65 | 0.29 | 0.30 | 0.41 | 0.41 | 0.34 | 0.33 | 0.49 |
|  | St Dev | 0.56 | 0.14 | 0.30 | 0.26 | 0.28 | 0.20 | 0.20 | 0.37 |
| Winter | Mean | 0.40 | 0.21 | 0.23 | 0.23 | 0.58/0.40 | 0.20 | 0.20 | 0.25 |
|  | St Dev | 0.67 | 0.11 | 0.12 | 0.10 | 1.71/0.32 | 0.16 | 0.29 | 0.24 |

(Without outliers)

Figure 4.  Graph showing the 68% confidence interval for NH3.

The abnormal standard deviation for w2 leads us to consider carefully the data for this site. Actually, we can see a value of 18 whereas all the other values are less than 4 (mean of 1.71 with this value 0.43 without). This value won’t be considered in the tests since it can only be a mistake.

We can then check the shape of the distribution of the logarithm of the concentration since we will use these data for the following analysis.

Table 3:  Frequency of log NH3 concentration

| Bin | S1 C | S8 C | S4 L | S6 L | S2 M | S7 M | S3 H | S5 H | W4 C | W7 C | W1 L | W6 L | W2 M | W7 M | W3 H | W5 H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -2.00 | 1 | 1 | 0 | 0 | 0 | 2 | 0 | 2 | 2 | 0 | 0 | 0 | 0 | 0 | 2 | 2 |
| -1.75 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -1.50 | 3 | 0 | 2 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |
| -1.25 | 1 | 1 | 3 | 0 | 2 | 1 | 1 | 2 | 1 | 2 | 2 | 2 | 0 | 2 | 5 | 3 |
| -1.00 | 0 | 4 | 2 | 4 | 1 | 4 | 4 | 1 | 8 | 13 | 5 | 7 | 5 | 13 | 19 | 14 |
| -0.75 | 4 | 11 | 8 | 13 | 8 | 18 | 8 | 5 | 22 | 28 | 28 | 24 | 22 | 28 | 19 | 22 |
| -0.50 | 34 | 47 | 40 | 25 | 30 | 33 | 43 | 24 | 52 | 56 | 56 | 48 | 35 | 56 | 42 | 62 |
| -0.25 | 14 | 35 | 40 | 31 | 35 | 36 | 23 | 29 | 21 | 14 | 17 | 20 | 30 | 14 | 2 | 15 |
| 0.00 | 22 | 4 | 2 | 19 | 12 | 14 | 7 | 19 | 4 | 0 | 5 | 0 | 11 | 0 | 0 | 4 |
| 0.25 | 14 | 0 | 0 | 2 | 3 | 0 | 2 | 8 | 5 | 0 | 0 | 0 | 10 | 0 | 0 | 1 |
| 0.50 | 5 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 2 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 0.75 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| More | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |

Figure 5.  Histogram of log concentration NH3 for summer plots.

Figure 6:  Histogram of log concentration NH3 for winter sites

#### ii) NOx

Table 4.  Means and standard deviation for NOx.

|  |  | Control |  | Low |  | Medium |  | High |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |
| Summer | Mean | 0.11/0.06 | 0.03 | 0.04 | 0.03 | 0.05 | 0.09 | 0.07 | 0.03 |
|  | St Dev | 0.29/0.15 | 0.07 | 0.11 | 0.10 | 0.13 | 0.29 | 0.23 | 0.07 |
| Winter | Mean | 0.12 | 0.07 | 0.27/0.13 | 0.08 | 0.38/0.10 | 0.03 | 0.04 | 0.12 |
|  | St Dev | 0.25 | 0.24 | 0.54/0.20 | 0.32 | 0.77/0.20 | 0.06 | 0.12 | 0.31 |

Figure 7.  Graph showing the 68% confidence interval for NOx.

The same approach for NOx reveals abnormal values in the site w2, which will be removed from the tests. It is very difficult to interpret the results since each site presents a great range (max value/min value about 100), but most of the cases, it is not matter of isolated outliers, just the natural spread of the datapoints. Besides, it is not reasonable to apply the Chauvenet’s criterion for outliers given the distribution, which is not Gaussian, unlike the other parameters (NH3, TP, TKN) are. Moreover, it would not be reasonable to get rid of 10% of the values.

Table 5.  Frequency of log NOx concentration.

| Bin | S1-C | S8-C | S4-L | S6-L | S2-M | S7-M | S3-H | S5-H | W4-C | W7-C | W1-L | W6-L | W2-M | W7-M | W3-H | W5-H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -2.00 | 62 | 72 | 74 | 60 | 61 | 69 | 63 | 72 | 42 | 56 | 44 | 50 | 42 | 58 | 50 | 51 |
| -1.80 | 2 | 5 | 3 | 2 | 3 | 8 | 1 | 1 | 6 | 6 | 3 | 6 | 9 | 6 | 8 | 6 |
| -1.60 | 9 | 8 | 5 | 15 | 10 | 10 | 11 | 5 | 16 | 18 | 7 | 17 | 8 | 18 | 17 | 8 |
| -1.40 | 4 | 6 | 2 | 4 | 4 | 4 | 1 | 5 | 8 | 5 | 5 | 2 | 6 | 5 | 5 | 11 |
| -1.20 | 4 | 5 | 5 | 4 | 4 | 3 | 5 | 2 | 10 | 11 | 6 | 10 | 5 | 11 | 3 | 13 |
| -1.00 | 2 | 3 | 1 | 4 | 3 | 5 | 0 | 2 | 2 | 5 | 10 | 4 | 2 | 5 | 2 | 6 |
| -0.80 | 3 | 0 | 2 | 2 | 1 | 1 | 1 | 0 | 11 | 4 | 2 | 4 | 7 | 4 | 2 | 4 |
| -0.60 | 4 | 0 | 1 | 1 | 1 | 2 | 1 | 2 | 7 | 1 | 11 | 3 | 6 | 1 | 1 | 11 |
| -0.40 | 1 | 3 | 0 | 1 | 1 | 1 | 0 | 3 | 4 | 2 | 5 | 2 | 6 | 2 | 1 | 6 |
| -0.20 | 4 | 1 | 3 | 0 | 3 | 1 | 3 | 0 | 8 | 3 | 6 | 1 | 5 | 3 | 1 | 3 |
| 0.00 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 2 | 0 | 2 | 0 | 3 | 0 | 1 | 2 |
| 0.20 | 2 | 0 | 0 | 0 | 0 | 3 | 2 | 0 | 2 | 0 | 9 | 1 | 7 | 0 | 0 | 2 |
| 0.40 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 4 | 1 | 0 | 0 |
| 0.60 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 1 | 5 | 0 | 0 | 1 |

Figure 8.  Histogram of log concentration NOx for summer sites.

Figure 9.  Histogram of log concentration NOx for winter sites.

Here, we must acknowledge that the concentration of NOx is not lognormally distributed. No need to say that Kolmogorov-Smirnov tests (which have been run for several sites) are unsuccessful. Indeed, the low values have really a great weight in the distribution. This means that the concentration is very often low, effectively zero. However, this property prevents us from being completely confident in the NOx GLM test, which requires a normal distribution.

#### iii) TKN

Table 6.  Means and standard deviation for TKN.

|  |  | Control |  | Low |  | Medium |  | High |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |
| Summer | Mean | 4.14 | 3.72 | 3.36 | 3.9 | 4.32 | 3.51 | 3.61 | 3.43 |
|  | St Dev | 1.95 | 1.58 | 1.32 | 1.93 | 1.71 | 1.84 | 1.46 | 2.02 |
| Winter | Mean | 3.07 | 3.12 | 2.66 | 2.98 | 3.49/3.32 | 2.55/2.62 | 2.78 | 2.82 |
|  | St Dev | 1.43 | 1.32 | 1.01 | 2.46 | 2.19/1.18 | 0.97/0.90 | 0.93 | 1.13 |

Figure 10.  Graph showing the 68% confidence interval for TKN.

Table 7.  Frequency of log TKN concentration.

| Bin | S1-C | S8-C | S4-L | S6-L | S2-M | S7-M | S3-H | S5-H | W4-C | W7-C | W1-L | W6-L | W2-M | W7-M | W3-H | W5-H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -1.70 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -1.50 | 0 | 4 | 0 | 2 | 0 | 0 | 0 | 2 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| -1.30 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -1.10 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -0.90 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -0.70 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -0.50 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| -0.30 | 0 | 0 | 1 | 1 | 0 | 4 | 0 | 2 | 1 | 0 | 0 | 1 | 0 | 0 | 2 | 0 |
| -0.10 | 0 | 1 | 2 | 2 | 0 | 1 | 0 | 3 | 1 | 4 | 0 | 2 | 0 | 4 | 0 | 1 |
| 0.10 | 8 | 0 | 5 | 4 | 3 | 2 | 6 | 2 | 9 | 8 | 15 | 13 | 9 | 8 | 3 | 9 |
| 0.30 | 7 | 10 | 12 | 12 | 4 | 23 | 5 | 16 | 21 | 14 | 19 | 16 | 8 | 14 | 7 | 24 |
| 0.50 | 12 | 11 | 13 | 8 | 13 | 18 | 21 | 15 | 34 | 28 | 31 | 19 | 34 | 28 | 52 | 38 |
| 0.70 | 38 | 57 | 54 | 43 | 41 | 36 | 38 | 33 | 46 | 51 | 47 | 43 | 56 | 51 | 26 | 49 |
| 0.90 | 28 | 18 | 9 | 22 | 29 | 23 | 18 | 18 | 6 | 9 | 0 | 6 | 8 | 9 | 1 | 3 |
| 1.10 | 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Figure 11.  Histogram of log concentration TKN for summer sites

We can see that the log-concentration of TKN is skewed, not symmetric and has more values on the left tail.

Figure 12.  Histogram of log concentration TKN for winter sites.

#### iv) TP

Table 8.  Means and standard deviation for TP

|  |  | Control |  | Low |  | Medium |  | High |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |
| Summer | ###### Mean | 1.07 | 0.82 | 1.11 | 0.85 | 0.88 | 0.7 | 0.79 | 0.8 |
|  | St Dev | 0.87 | 0.57 | 0.98 | 0.88 | 0.5 | 0.76 | 0.5 | 0.98 |
| Winter | Mean | 0.11 | 0.15 | 0.12 | 0.18 | 0.14 | 0.08 | 0.24 | 0.16 |
|  | St Dev | 0.13 | 0.16 | 0.09 | 0.17 | 0.20 | 0.10 | 0.27 | 0.25 |

Figure 13.  Graph showing the 68% confidence interval for TP.

It is obvious here that the summer pasture nutrient concentrations are higher than the winter pasture concentrations (factor of 5 or more).

Table 9.  Frequency of log TP concentration.

| Bin | S1C | S8C | S4L | S6L | S2M | S7M | S3H | S5H | W4C | W7C | W1L | W6L | W2M | W7M | W3H | W5H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -2.70 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -2.45 | 0 | 2 | 0 | 2 | 0 | 0 | 0 | 3 | 4 | 0 | 1 | 1 | 3 | 4 | 3 | 1 |
| -2.20 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -1.95 | 0 | 2 | 1 | 0 | 1 | 2 | 0 | 0 | 2 | 1 | 8 | 1 | 0 | 2 | 2 | 0 |
| -1.70 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 0 | 2 | 0 | 2 | 2 | 3 | 3 |
| -1.45 | 1 | 2 | 0 | 0 | 0 | 2 | 1 | 1 | 13 | 6 | 5 | 1 | 5 | 18 | 3 | 3 |
| -1.20 | 3 | 0 | 0 | 1 | 0 | 3 | 0 | 3 | 33 | 33 | 18 | 21 | 18 | 30 | 11 | 29 |
| -0.95 | 8 | 1 | 1 | 2 | 3 | 6 | 9 | 3 | 30 | 25 | 29 | 25 | 45 | 35 | 10 | 31 |
| -0.70 | 5 | 7 | 6 | 10 | 5 | 14 | 1 | 7 | 21 | 18 | 31 | 14 | 25 | 15 | 12 | 33 |
| -0.45 | 18 | 10 | 18 | 18 | 12 | 18 | 13 | 24 | 4 | 26 | 17 | 27 | 11 | 4 | 30 | 14 |
| -0.20 | 11 | 27 | 13 | 12 | 13 | 20 | 22 | 20 | 9 | 4 | 2 | 10 | 3 | 0 | 14 | 7 |
| 0.05 | 8 | 21 | 20 | 29 | 21 | 22 | 13 | 6 | 0 | 0 | 0 | 0 | 2 | 1 | 2 | 2 |
| 0.30 | 21 | 28 | 27 | 13 | 37 | 16 | 30 | 18 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |
| 0.55 | 23 | 2 | 6 | 2 | 0 | 4 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 0.80 | 0 | 0 | 5 | 5 | 0 | 2 | 0 | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Figure 14.  Histogram of log concentration TP for summer sites.

Like TKN, We can see that the log-concentration of TP is not symmetric and has more values on the left tail.

Figure 15.  Histogram of log concentration TP for winter sites.

It is interesting to note that the log-concentration of TP for winter has a good bell shape (except for w3).

### b) Duplicate tests

#### Year 2001

Here are the results of the tests to verify if the replicate means are similar. If the P-value is below 5%, it can be concluded that the 2 means cannot be considered as equal.

Table 10.  P-value of the T-Tests for replicate sites (2001).

|  | Summer |  |  |  | Winter |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Control | Low | Medium | High | Control | Low | Medium | High |
| NH3 | 0.000 | 0.002 | 0.004 | 0.002 | 0.003 | 0.429 | 0.000 | 0.161 |
| NOx | 0.011 | 0.710 | 0.219 | 0.081 | 0.066 | 0.003 | 0.000 | 0.015 |
| TKN | 0.082 | 0.027 | 0.002 | 0.564 | 0.767 | 0.076 | 0.245 | 0.845 |
| TP | 0.016 | 0.067 | 0.053 | 0.863 | 0.073 | 0.000 | 0.004 | 0.032 |

This shows that we must be very cautious drawing conclusions since for half of the sites (17 / 32), it is impossible to know what the real effect is. However, the ANOVA GLM procedure takes this into account.

#### Year 1999

Table 11.  P-value of the T-Tests for replicate sites (1999).

|  | Summer |  |  |  | Winter |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Control | Low | Medium | High | Control | Low | Medium | High |
| NH3 | 0.066 | 0.476 | 0.723 | 0.144 | 0.627 | 0.071 | 0.457 | 0.670 |
| NOx | 0.097 | 0.461 | 0.775 | 0.047 | 0.242 | 0.016 | 0.169 | 0.000 |
| TKN | 0.184 | 0.539 | 0.435 | 0.960 | 0.631 | 0.402 | 0.323 | 0.836 |
| TP | 0.067 | 0.477 | 0.723 | 0.144 | 0.627 | 0.071 | 0.458 | 0.670 |

The table 11 shows that the replicates for 1999 are correct and give the same means for a same treatment except in 2 cases.

#### Year 2000

Table 12.  P-value of the T-Tests for replicate sites (2000).

|  | Summer |  |  |  | Winter |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Control | Low | Medium | High | Control | Low | Medium | High |
| NH3 | 0.847 | 0.489 | 0.111 | 0.108 | 0.619 | 0.979 | 0.503 | 0.150 |
| NOx | 0.350 | 1.000 | 0.341 | 1.000 | 0.919 | 0.796 | 0.439 | 0.391 |
| TKN | 0.001 | 0.301 | 0.012 | 0.108 | 0219 | 0.327 | 0.822 | 0.138 |
| TP | 0.000 | 0.845 | 0.443 | 0.037 | 0.008 | 0.019 | 0.347 | 0.288 |

We can see on the table 12 that replicates for 2000 are quite good for NH3 and TKN. For TP and to a lesser extent for TKN, we can underline a difference between the means for several treatments.

### c) GLM tests

#### Year 2001

Let’s consider that we can run the tests. The following table shows the results for the four nutrients. (The Minitab window is shown in the Appendix)

Table 13.  P-value of the GLM tests (2001).

|  | NH3 | NO x | TKN | TP |
| --- | --- | --- | --- | --- |
| Block | 0.000 | 0.000 | 0.000 | 0.000 |
| Treatment | 0.056 | 0.313 | 0.000 | 0.004 |
| B * T | 0.010 | 0.138 | 0.000 | 0.001 |

Different conclusions can be drawn:

- The block effect is always statistically significant. It means that summer and winter treatments have an influence on the nutrient concentrations.

- For TKN and TP, the treatment effect is also significant. Concentrations differ with the treatment (control / low / medium / high).

- For NOx, the very high P-value (31%) allows us to conclude that the concentration do not differ with the treatment

- It is more difficult to conclude for NH3 since the value (5.6%) is closed to the threshold (5%). We can say that the treatment effect is not very significant but that the influence is yet more important than for NOx.

#### Year 1999

The GLM method has not been applied for the data of the previous years. To complement the report, we can insert here the results for 1999.

Table 14.  P-value of the GLM tests (1999).

|  | NH3 | NO x | TKN | TP |
| --- | --- | --- | --- | --- |
| Block | 0.000 | 0.001 | 0.000 | 0.000 |
| Treatment | 0.000 | 0.002 | 0.970 | 0.883 |
| B * T | 0.117 | 0.004 | 0.027 | 0.638 |

Conclusions:

- The block effect is always statistically significant. It means that summer and winter treatments have an influence on the nutrient concentrations.

- For NH3 and NOx, the treatment effect is also significant. Concentrations differ with the treatment (control / low / medium / high).

- For TKN and TP, the very high P-value (97% and 88%) allows us to conclude that the concentration do not differ with the treatment

#### Year 2000

Table 15.  P-value of the GLM tests (2000).

|  | NH3 | NO x | TKN | TP |
| --- | --- | --- | --- | --- |
| Block | 0.000 | 0.000 | 0.000 | 0.864 |
| Treatment | 0.695 | 0.028 | 0.471 | 0.018 |
| B * T | 0.204 | 0.121 | 0.374 | 0.560 |

Conclusions:

- The block effect is often statistically significant (for all the nutrients except TP). It means that summer and winter treatments have an influence on the nutrient concentrations.

- There is no significant effect for any of the treatments

### d) Concentration vs. Treatment

#### Year 2001

The previous test showed that in most of the cases, the treatments had an effect on the nutrient concentrations. In other cases the variability of the data could not allow drawing such a conclusion.

So, let’s consider the TP concentration data, which satisfy the first assumption (lognormal distribution). We can report on a graph the concentration vs. the number of cow-calf by unit. Unfortunately, we do not obtain monotonous curves. Therefore, it is impossible to conclude on the nature of the effect although the apparent relationship is actually inverse. The study of the other nutrients leads to the same conclusions:

Figure 16.  TP concentration vs. treatment.

Figure 17.  TKN concentration vs. treatment.

Figure 18.  NOx concentration vs. treatment.

Figure 19.  NH3 concentration vs. treatment.

#### Year 1999

Table 16.  Means of concentration for each treatment.

|  |  | Control | Low | Medium | High |
| --- | --- | --- | --- | --- | --- |
|  |  | 0 | 15 | 20 | 35 |
| NH3 | Summer | 0.305 | 0.365 | 0.500 | 0.425 |
|  | Winter | 0.280 | 0.220 | 0.215 | 0.320 |
| NOx | Summer | 0.011 | 0.009 | 0.010 | 0.003 |
|  | Winter | 0.017 | 0.025 | 0.018 | 0.028 |
| TKN | Summer | 4.310 | 6.500 | 8.670 | 5.590 |
|  | Winter | 4.290 | 4.385 | 3.055 | 4.655 |
| TP | Summer | 0.656 | 0.765 | 1.130 | 0.795 |
|  | Winter | 0.209 | 0.140 | 0.170 | 0.148 |

Figure 20.  NH3 concentration vs. treatment.

Figure 21.  NOx concentration vs. treatment.

Figure 22.  TKN concentration vs. treatment.

Figure 23.  TP concentration vs. treatment.

#### Year 2000

Table 17.  Means of concentration for each treatment.

|  |  | Control | Low | Medium | High |
| --- | --- | --- | --- | --- | --- |
|  |  | 0 | 15 | 20 | 35 |
| NH3 | Summer | 0.275 | 0.128 | 0.099 | 0.112 |
|  | Winter | 0.466 | 0.7373 | 0.7979 | 1.291 |
| NOx | Summer | 0.011 | 0.01 | 0.025 | 0.01 |
|  | Winter | 0.1165 | 0.0257 | 0.095 | 0.075 |
| TKN | Summer | 2.65 | 2.71 | 2.058 | 2.272 |
|  | Winter | 3.5235 | 4.6 | 4.2 | 5.1435 |
| TP | Summer | 0.402 | 0.277 | 0.184 | 0.212 |
|  | Winter | 0.181 | 0.5262 | 0.118 | 0.393 |

Figure 24.  NH3 concentration vs. treatment.

Figure 25.  NOx concentration vs. treatment.

Figure 26.  TKN concentration vs. treatment.

Figure 27.  TP concentration vs. treatment.

## V. Tukey’s w test

### Objective

So far, we carried out global tests in order to identify if a block or a treatment effect dould be found, analyzing, analyzing the whole data (16 sites). It is possible to process a more advanced study, analyzing all the treatments by pairs. Let’s consider the Tukey’s w procedure. It makes use of the Studentized range and is applicable to pairwise comparisons of means. It requires a single value for judging the significance of all differences and is thus quick and easy to use. However, this test can only consider one factor at a time (it is a one-way ANOVA test). It is the reason why we will apply it only for the sites within a block (summer/winter).

### Tukey’s w procedure on Minitab

MS Excel cannot carry out this kind of tests. Therefore, Minitab was used instead. Just as we did for the GLM test, we put all the data in a column with the treatment corresponding in another one. Obviously, the 2 columns must have the same length.

Then, we just select Stat / ANOVA / One-way and specify the option Comparison / Tukey’s with a family error rate of 5%.

Therefore, we obtain in the Session window the results, which consist on:

- A one-way analysis of variance for all the data

- A table with the means, standard deviations and confidence interval

- The Tukey’s test

The first pair of number in the Tukey output table gives the confidence interval for the difference between the mean of Column and the mean of Row (for instance for the first test: mean of concentration of S1 – idem for S8). [See the first table]. If 0 is not included in this interval (when the 2 values have the same sign), it means that the means can be declared different (since the difference cannot be null).

If it is the case for 2 means in a same treatment, it means that 2 replications present different averages. Obviously, it is bad for our study since it shows we cannot trust the treatment effect. Other parameters we cannot identify have an effect on the concentrations. Don’t forget that a negative result shows that 2 means are significantly different but a positive result do not show that the 2 means are equal!

Besides, the case when the values have the same sign is expected for sites of different treatments, otherwise, we can also say that the variability of the data avoids knowing which the higher value among the two of them is.

### Tests

Here are presented the results of the Tukey’s w procedure.

| 0 | We cannot compare the 2 means |
| --- | --- |
| + | Mean of column > Mean of row |
| - | Mean of column < Mean of row |

Therefore, an experiment for which the replicates gave the same means and for which it is possible to see a significant difference between the treatments have the following results (given that the more the treatment is important, the higher the concentration should be):

Table 18.  Tukey's test - Example.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Example |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | 0 |  |  |  |  |  |  |
| Low | S4 | + | + |  |  |  |  |  |
|  | S6 | + | + | 0 |  |  |  |  |
| Medium | S2 | + | + | + | + |  |  |  |
|  | S7 | + | + | + | + | 0 |  |  |
| High | S3 | + | + | + | + | + | + |  |
|  | S5 | + | + | + | + | + | + | 0 |

#### Tukey’s procedure – 1999

Table 19.  Tukey's test - Summer pasture - NH3 – 1999.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NH3 - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | 0 |  |  |  |  |  |  |
| Low | S4 | 0 | 0 |  |  |  |  |  |
|  | S6 | + | 0 | 0 |  |  |  |  |
| Medium | S2 | 0 | - | 0 | - |  |  |  |
|  | S7 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | S3 | 0 | 0 | 0 | - | 0 | 0 |  |
|  | S5 | 0 | 0 | 0 | 0 | + | 0 | 0 |

Table 20.  Tukey's test - Winter pasture – NH3 – 1999.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NH3 - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | 0 |  |  |  |  |  |  |
| Low | W1 | + | 0 |  |  |  |  |  |
|  | W6 | 0 | 0 | 0 |  |  |  |  |
| Medium | W2 | 0 | 0 | 0 | 0 |  |  |  |
|  | W8 | 0 | 0 | - | 0 | 0 |  |  |
| High | W3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | W5 | 0 | 0 | - | 0 | - | 0 | 0 |

Table 21.  Tukey's test - Summer pasture - NOx – 1999.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOx - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | 0 |  |  |  |  |  |  |
| Low | S4 | 0 | 0 |  |  |  |  |  |
|  | S6 | 0 | 0 | 0 |  |  |  |  |
| Medium | S2 | 0 | 0 | 0 | 0 |  |  |  |
|  | S7 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | S3 | 0 | 0 | 0 | 0 | + | + |  |
|  | S5 | 0 | 0 | 0 | 0 | + | + | 0 |

Table 22.  Tukey's test - Winter pasture - NOx – 1999.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOx - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | 0 |  |  |  |  |  |  |
| Low | W1 | - | - |  |  |  |  |  |
|  | W6 | 0 | 0 | + |  |  |  |  |
| Medium | W2 | 0 | 0 | 0 | 0 |  |  |  |
|  | W8 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | W3 | 0 | 0 | + | 0 | 0 | 0 |  |
|  | W5 | 0 | 0 | 0 | 0 | 0 | 0 | - |

Table 23.  Tukey's test - Summer pasture - TKN – 1999.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TKN - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | 0 |  |  |  |  |  |  |
| Low | S4 | 0 | 0 |  |  |  |  |  |
|  | S6 | 0 | 0 | 0 |  |  |  |  |
| Medium | S2 | 0 | 0 | 0 | 0 |  |  |  |
|  | S7 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | S3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | S5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 24.  Tukey's test - Winter pasture - TKN – 1999.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TKN - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | 0 |  |  |  |  |  |  |
| Low | W1 | 0 | 0 |  |  |  |  |  |
|  | W6 | 0 | 0 | 0 |  |  |  |  |
| Medium | W2 | 0 | 0 | 0 | 0 |  |  |  |
|  | W8 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | W3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | W5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 25.  Tukey's test - Summer pasture - TP – 1999.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TP - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | 0 |  |  |  |  |  |  |
| Low | S4 | 0 | 0 |  |  |  |  |  |
|  | S6 | 0 | 0 | 0 |  |  |  |  |
| Medium | S2 | 0 | 0 | 0 | 0 |  |  |  |
|  | S7 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | S3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | S5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 26.  Tukey's test - Winter pasture - TP – 1999.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TP - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | 0 |  |  |  |  |  |  |
| Low | W1 | 0 | 0 |  |  |  |  |  |
|  | W6 | 0 | 0 | 0 |  |  |  |  |
| Medium | W2 | 0 | 0 | 0 | 0 |  |  |  |
|  | W8 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | W3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | W5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

#### Tukey’s procedure – 2000

Table 27.  Tukey's test - Summer pasture - NH3 – 2000.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NH3 - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | 0 |  |  |  |  |  |  |
| Low | S4 | 0 | 0 |  |  |  |  |  |
|  | S6 | 0 | 0 | 0 |  |  |  |  |
| Medium | S2 | 0 | 0 | 0 | 0 |  |  |  |
|  | S7 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | S3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | S5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 28.  Tukey's test - Winter pasture - NH3 – 2000.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NH3 - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | 0 |  |  |  |  |  |  |
| Low | W1 | 0 | 0 |  |  |  |  |  |
|  | W6 | 0 | 0 | 0 |  |  |  |  |
| Medium | W2 | 0 | 0 | 0 | 0 |  |  |  |
|  | W8 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | W3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | W5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 29.  Tukey's test - Sumer pasture - NOx – 2000.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOx - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | 0 |  |  |  |  |  |  |
| Low | S4 | 0 | 0 |  |  |  |  |  |
|  | S6 | 0 | 0 | 0 |  |  |  |  |
| Medium | S2 | 0 | 0 | 0 | 0 |  |  |  |
|  | S7 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | S3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | S5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 30.  Tukey's test - Winter pasture - NOx – 2000.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOx - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | 0 |  |  |  |  |  |  |
| Low | W1 | 0 | 0 |  |  |  |  |  |
|  | W6 | 0 | 0 | 0 |  |  |  |  |
| Medium | W2 | 0 | 0 | 0 | 0 |  |  |  |
|  | W8 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | W3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | W5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 31.  Tukey's test - Summer pasture - TKN – 2000.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TKN - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | - |  |  |  |  |  |  |
| Low | S4 | 0 | 0 |  |  |  |  |  |
|  | S6 | - | 0 | 0 |  |  |  |  |
| Medium | S2 | 0 | 0 | 0 | 0 |  |  |  |
|  | S7 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | S3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | S5 | - | 0 | 0 | 0 | 0 | 0 | 0 |

Table 32.  Tukey's test - Winter pasture - TKN – 2000.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TKN - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | 0 |  |  |  |  |  |  |
| Low | W1 | 0 | 0 |  |  |  |  |  |
|  | W6 | 0 | 0 | 0 |  |  |  |  |
| Medium | W2 | 0 | 0 | 0 | 0 |  |  |  |
|  | W8 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | W3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | W5 | 0 | 0 | 0 | 0 | 0 | 0 | - 0 |

Table 33.  Tukey's test - Summer pasture - TP – 2000.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TP - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | - |  |  |  |  |  |  |
| Low | S4 | - | 0 |  |  |  |  |  |
|  | S6 | - | 0 | 0 |  |  |  |  |
| Medium | S2 | 0 | 0 | 0 | 0 |  |  |  |
|  | S7 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | S3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | S5 | - | 0 | 0 | 0 | 0 | 0 | 0 |

Table 34.  Tukey's test - Winter pasture - TP – 2000.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TP - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | 0 |  |  |  |  |  |  |
| Low | W1 | 0 | 0 |  |  |  |  |  |
|  | W6 | - | 0 | - |  |  |  |  |
| Medium | W2 | 0 | 0 | 0 | + |  |  |  |
|  | W8 | 0 | 0 | 0 | + | 0 |  |  |
| High | W3 | 0 | 0 | 0 | 0 | 0 | - | 0 |
|  | W5 | 0 | 0 | 0 | + | 0 | 0 | 0 |

#### Tukey’s procedure – 2001

Table 35.  Tukey's test - Summer pasture - NH3 – 2001.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NH3 - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | + | 0 |  |  |  |  |  |
| Low | S4 | + | 0 |  |  |  |  |  |
|  | S6 | 0 | 0 | 0 |  |  |  |  |
| Medium | S2 | 0 | 0 | 0 | 0 |  |  |  |
|  | S7 | + | 0 | 0 | 0 | 0 |  |  |
| High | S3 | + | 0 | 0 | 0 | 0 | 0 |  |
|  | S5 | 0 | + | 0 | 0 | 0 | 0 | 0 |

Table 36.  Tukey's test - Winter pasture - NH3 – 2001.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NH3 - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | + |  |  |  |  |  |  |
| Low | W1 | 0 | 0 |  |  |  |  |  |
|  | W6 | 0 | 0 | 0 |  |  |  |  |
| Medium | W2 | 0 | + | 0 | + |  |  |  |
|  | W8 | + | 0 | 0 | 0 | + |  |  |
| High | W3 | - | 0 | + | - | + | 0 |  |
|  | W5 | 0 | 0 | 0 | 0 | + | 0 | 0 |

Table 37.  Tukey's test - Summer pasture - NOx – 2001.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOx - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | 0 |  |  |  |  |  |  |
| Low | S4 | 0 | 0 |  |  |  |  |  |
|  | S6 | 0 | 0 | 0 |  |  |  |  |
| Medium | S2 | 0 | 0 | 0 | 0 |  |  |  |
|  | S7 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | S3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | S5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 38.  Tukey's test - Winter pasture - NOx – 2001.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOx - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | + |  |  |  |  |  |  |
| Low | W1 | 0 | + |  |  |  |  |  |
|  | W6 | 0 | 0 | + |  |  |  |  |
| Medium | W2 | 0 | + | 0 | + |  |  |  |
|  | W8 | + | 0 | + | 0 | + |  |  |
| High | W3 | 0 | 0 | + | 0 | + | 0 |  |
|  | W5 | 0 | 0 | 0 | 0 | 0 | + | - |

Table 39.  Tukey's test - Summer pasture - TKN – 2001.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TKN - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | 0 |  |  |  |  |  |  |
| Low | S4 | 0 | 0 |  |  |  |  |  |
|  | S6 | 0 | 0 | 0 |  |  |  |  |
| Medium | S2 | 0 | 0 | 0 | 0 |  |  |  |
|  | S7 | 0 | 0 | 0 | 0 | 0 |  |  |
| High | S3 | 0 | 0 | 0 | - | 0 | 0 |  |
|  | S5 | 0 | 0 | 0 | 0 | + | 0 | 0 |

Table 40.  Tukey's test - Winter pasture - TKN – 2001.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TKN - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | 0 |  |  |  |  |  |  |
| Low | W1 | 0 | 0 |  |  |  |  |  |
|  | W6 | 0 | 0 | 0 |  |  |  |  |
| Medium | W2 | 0 | 0 | - | 0 |  |  |  |
|  | W8 | 0 | + | 0 | 0 | + |  |  |
| High | W3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | W5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 41.  Tukey's test - Summer pasture - TP – 2001.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TP - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | 0 |  |  |  |  |  |  |
| Low | S4 | 0 | 0 |  |  |  |  |  |
|  | S6 | 0 | 0 | 0 |  |  |  |  |
| Medium | S2 | 0 | 0 | 0 | 0 |  |  |  |
|  | S7 | 0 | 0 | + | 0 | 0 |  |  |
| High | S3 | 0 | 0 | 0 | 0 | 0 | 0 |  |
|  | S5 | 0 | 0 | + | 0 | 0 | 0 | 0 |

Table 42.  Tukey's test - Winter pasture - TP – 2001.

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TP - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | - |  |  |  |  |  |  |
| Low | W1 | 0 | 0 |  |  |  |  |  |
|  | W6 | - | 0 | 0 |  |  |  |  |
| Medium | W2 | 0 | 0 | 0 | 0 |  |  |  |
|  | W8 | 0 | + | + | + | + |  |  |
| High | W3 | + | 0 | - | 0 | 0 | + |  |
|  | W5 | - | 0 | 0 | 0 | 0 | + | 0 |

#### Conclusion

I considered as useless the fact of drawing a conclusion for each test. Indeed, all the results go along the same lines:

- The majority of the cells are null, what it means that the distributions do not allow comparing two means each other.

- There is not a majority of positive results compared to negative ones. It states that a more important treatment increase a nutrient concentration sometimes, decrease it other times.

Therefore, all the different conclusions agree to say that the data of 1999/2000/2001 are incapable to bring to the fore a difference between the 4 different treatments: control, low, medium, high. Consequently:

- Either it does exist an influence but the experiment has not managed to unveil it, because of experiment errors or simply because the signal is lost in a noise of high amplitude.

- Either it must be admitted that the density of cow/calf by unit has no effect on the concentration of the 4 nutrients concerned.

## Conclusion

This paper brings to the fore the difficulty of drawing conclusions for the Buck Island Project. First of all, we have seen that the data contained several anomalies, which revealed experimental errors (measuring the parameter) or computer errors (reporting the value in the database) mistakes. Nevertheless, after eliminating the outliers the statistical tests were performed. Then, the different conclusions must be drawn very cautiously because all the distributions used did not respect the normal distribution. Afterwards, we realized that means, which were supposed to be equal, are not, nearly for half of them, so that it is difficult to figure out what really happens (Sometimes, we have a ratio of 10 between two means representing the same treatment!). Lastly, even if the GLM tests state that an effect for a nutrient can be identified, it is less obvious to find the nature of this effect given that the curves of the concentration vs. the treatments are not monotonous (increasing curves were expected).

## Appendix 1: GLM tests with Minitab (1999)

Factor     Type Levels Values

Block     fixed      2 summer winter

Treatment fixed      4 control high    low     medium

Analysis of Variance for NH3, using Adjusted SS for Tests

Source           DF     Seq SS     Adj SS     Adj MS       F      P

Block             1    5.57339    4.90896    4.90896  122.67  0.000

Treatment         3    0.88935    0.82794    0.27598    6.90  0.000

Block*Treatment   3    0.23691    0.23691    0.07897    1.97  0.117

Error           729   29.17291   29.17291    0.04002

Analysis of Variance for NOx, using Adjusted SS for Tests

Source           DF     Seq SS     Adj SS     Adj MS       F      P

Block             1    0.63091    0.91773    0.91773   11.09  0.001

Treatment         3    1.42231    1.19704    0.39901    4.82  0.002

Block*Treatment   3    1.13306    1.13306    0.37769    4.56  0.004

Error           731   60.49710   60.49710    0.08276

Total           738   63.68338

Analysis of Variance for TKN, using Adjusted SS for Tests

Source           DF     Seq SS     Adj SS     Adj MS       F      P

Block             1    2.59755    2.55429    2.55429   34.07  0.000

Treatment         3    0.05938    0.01850    0.00617    0.08  0.970

Block*Treatment   3    0.69137    0.69137    0.23046    3.07  0.027

Error           728   54.58008   54.58008    0.07497

Total           735   57.92837

Analysis of Variance for TP, using Adjusted SS for Tests

Source           DF     Seq SS     Adj SS     Adj MS       F      P

Block             1    119.906    114.569    114.569  810.40  0.000

Treatment         3      0.071      0.093      0.031    0.22  0.883

Block*Treatment   3      0.240      0.240      0.080    0.57  0.638

Error           729    103.060    103.060      0.141

Total           736    223.277

## Appendix 2: GLM tests with Minitab (2000)

Factor     Type Levels Values

Block     fixed      2 summer winter

Treatment fixed      4 control high    low     medium

Analysis of Variance for NH3, using Adjusted SS for Tests

Source           DF     Seq SS     Adj SS     Adj MS       F      P

Block             1    11.5341    11.5655    11.5655   48.36  0.000

Treatment         3     0.3635     0.3468     0.1156    0.48  0.695

Block*Treatment   3     1.1206     1.1206     0.3735    1.56  0.204

Error            87    20.8044    20.8044     0.2391

Total            94    33.8226

Analysis of Variance for NOx, using Adjusted SS for Tests

Source           DF     Seq SS     Adj SS     Adj MS       F      P

Block             1     4.1344     4.0470     4.0470   21.31  0.000

Treatment         3     1.2505     1.8139     0.6046    3.18  0.028

Block*Treatment   3     1.1362     1.1362     0.3787    1.99  0.121

Error            87    16.5216    16.5216     0.1899

Total            94    23.0426

Analysis of Variance for TKN, using Adjusted SS for Tests

Source           DF     Seq SS     Adj SS     Adj MS       F      P

Block             1    1.05733    0.93519    0.93519   13.22  0.000

Treatment         3    0.14164    0.18021    0.06007    0.85  0.471

Block*Treatment   3    0.22337    0.22337    0.07446    1.05  0.374

Error            87    6.15329    6.15329    0.07073

Total            94    7.57563

Analysis of Variance for TP, using Adjusted SS for Tests

Source           DF     Seq SS     Adj SS     Adj MS       F      P

Block             1     0.0068     0.0058     0.0058    0.03  0.864

Treatment         3     1.8215     2.0574     0.6858    3.52  0.018

Block*Treatment   3     0.4041     0.4041     0.1347    0.69  0.560

Error            87    16.9694    16.9694     0.1951

Total            94    19.2017

## Appendix 3: GLM Test with Minitab (2001)

Factor     Type Levels Values

Block     fixed      2 summer winter

Treatment fixed      4 control high    low     medium

###### Analysis of Variance for NH3_log, using Adjusted SS for Tests

Source          DF     Seq SS     Adj SS     Adj MS       F      P

Block            1    12.4749    12.5995    12.5995  119.27  0.000

Treatment        3     0.9325     0.7995     0.2665    2.52  0.056

Block*Treatment  3     1.2111     1.2111     0.4037    3.82  0.010

Error         1653   174.6262   174.6262     0.1056

Total         1660   189.2447

###### Analysis of Variance for NOx_log, using Adjusted SS for Tests

Source          DF     Seq SS     Adj SS     Adj MS       F      P

Source          DF     Seq SS     Adj SS     Adj MS       F      P

Block            1    24.3559    24.6486    24.6486   77.02  0.000

Treatment        3     1.2503     1.1405     0.3802    1.19  0.313

Block*Treatment  3     1.7684     1.7684     0.5895    1.84  0.138

Error         1653   528.9825   528.9825     0.3200

Total         1660   556.3571

###### Analysis of Variance for TKN_log, using Adjusted SS for Tests

Source          DF     Seq SS     Adj SS     Adj MS       F      P

Block             1     48.321     46.815     46.815  169.17  0.000

Treatment         3     93.519     81.524     27.175   98.20  0.000

Block*Treatment   3     93.588     93.588     31.196  112.73  0.000

Error          1652    457.173    457.173      0.277

Total          1659    692.601

###### Analysis of Variance for TP_log, using Adjusted SS for Tests

Source          DF     Seq SS     Adj SS     Adj MS       F      P

Block             1    247.353    245.141    245.141 1085.14  0.000

Treatment         3      3.332      3.032      1.011    4.47  0.004

Block*Treatment   3      3.611      3.611      1.204    5.33  0.001

Error          1653    373.425    373.425      0.226

Total          1660    627.721

## Appendix 4

Tukey’s test – NH3 – Summer - 1999

Source     DF        SS        MS        F        P

S           7    1.1349    0.1621     5.36    0.000

Error     381   11.5143    0.0302

Total     388   12.6491

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  -------+---------+---------+---------

1          50   -0.5175    0.1721                  (-----*-----)

2          45   -0.4797    0.1400                       (-----*-----)

3          29   -0.5221    0.2896                (-------*-------)

4          44   -0.5632    0.1838            (------*-----)

5          45   -0.6170    0.1841      (-----*-----)

6          57   -0.6493    0.1560  (-----*----)

0.2078      0.2484      0.2204      0.1656-)

8          52   -0.6139    0.1543      (-----*-----)

-------+---------+---------+---------

Pooled StDev =   0.1738                -0.640    -0.560    -0.480

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00258

Critical value = 4.29

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6          7

2 -0.1461

0.0706

3 -0.1185     -0.0832

0.1277      0.1680

4 -0.0633     -0.0283     -0.0851

0.1547      0.1953      0.1672

5 -0.0089      0.0261     -0.0307     -0.0580

0.2078      0.2484      0.2204      0.1656

6  0.0296      0.0644      0.0069     -0.0197     -0.0729

0.2340      0.2747      0.2474      0.1919      0.1375

7 -0.0538     -0.0191     -0.0771     -0.1033     -0.1564     -0.1820

0.1433      0.1842      0.1574      0.1014      0.0469      0.0080   -0.0458

8 -0.0080      0.0268     -0.0304     -0.0573     -0.1104     -0.1365   -0.0458

0.2009      0.2415      0.2140      0.1587      0.1043      0.0658    0.1491

Tukey’s test – NOx – Summer - 1999

Source     DF        SS        MS        F        P

S           7    1.5997    0.2285     4.53    0.000

Error     383   19.3157    0.0504

Total     390   20.9154

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  --------+---------+---------+--------

1          50   -1.8242    0.2117                     (------*-----)

2          45   -1.7801    0.2707                         (------*------)

3          29   -1.9896    0.0559   (-------*-------)

8          53   -1.8969    0.2291              (-----*-----)

5          45   -1.9444    0.1324         (------*-----)

6          57   -1.8751    0.2367                 (----*-----)

7          67   -1.7955    0.2906                         (----*-----)

8          53   -1.8969    0.2291              (-----*-----)

--------+---------+---------+--------

Pooled StDev =   0.2246                 -2.00     -1.90     -1.80

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00258

Critical value = 4.29

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6        7

2  -0.1840

0.0959

3   0.0064      0.0473

0.3244      0.3717

4  -0.0578     -0.0174     -0.2455

0.2221      0.2698      0.0789

5  -0.0198      0.0207     -0.2074     -0.1055

0.2602      0.3079      0.1170      0.1817

6  -0.0811     -0.0409     -0.2699     -0.1671     -0.2052

0.1829      0.2308      0.0408      0.1046      0.0665

7  -0.1560     -0.1159     -0.3455     -0.2421     -0.2802     -0.2023

0.0986      0.1467     -0.0427      0.0205     -0.0176      0.0432

8  -0.0617     -0.0214     -0.2501     -0.1476     -0.1857     -0.1082  -0.0239

0.2070      0.2548      0.0646      0.1286      0.0905      0.1518   0.2265

Tukey’s test – TKN – Summer - 1999

Source     DF        SS        MS        F        P

S           7    0.6218    0.0888     1.29    0.252

Error     380   26.0863    0.0686

Total     387   26.7081

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  ----------+---------+---------+------

1          50    0.6403    0.1472             (--------*--------)

2          45    0.6839    0.0993                  (--------*---------)

3          28    0.5964    0.2703    (------------*-----------)

4          45    0.5641    0.2798   (---------*--------)

5          44    0.5999    0.3105       (---------*---------)

7          66    0.6598    0.2194                 (------*-------)

8          53    0.5721    0.3379     (--------*-------)

----------+---------+---------+------

Pooled StDev =   0.2620                    0.560     0.640     0.720

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00258

Critical value = 4.29

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6       7

2  -0.2069

0.1197

3  -0.1437     -0.1038

0.2315      0.2788

4  -0.0871     -0.0477     -0.1590

0.2396      0.2874      0.2236

5  -0.1239     -0.0845     -0.1957     -0.2044

0.2047      0.2525      0.1886      0.1327

6  -0.1147     -0.0756     -0.1881     -0.1955     -0.1606

0.1933      0.2414      0.1788      0.1215      0.1584

7  -0.1685     -0.1295     -0.2426     -0.2494     -0.2145     -0.2024

0.1296      0.1778      0.1159      0.0579      0.0948      0.0850

8  -0.0885     -0.0493     -0.1614     -0.1691     -0.1343     -0.1227  -0.0589

0.2249      0.2730      0.2100      0.1531      0.1899      0.1806   0.2343

Tukey’s test – TP – Summer - 1999

Source     DF        SS        MS        F        P

S           7     0.945     0.135     0.82    0.575

Error     381    63.117     0.166

Total     388    64.063

Individual 95% CIs For Mean

3          28   -0.4637    0.5062  (---------*---------)

Level       N      Mean     StDev  -+---------+---------+---------+-----

1          49   -0.3688    0.3963           (------*-------)

2          45   -0.3312    0.3362             (-------*-------)

3          28   -0.4637    0.5062  (---------*---------)

4          45   -0.3865    0.4409         (-------*-------)

5          45   -0.3259    0.4862             (-------*-------)

6          57   -0.3308    0.3167              (------*------)

7          67   -0.3581    0.4637             (-----*------)

8          53   -0.2583    0.3077                  (-------*------)

-+---------+---------+---------+-----

Pooled StDev =   0.4070          -0.60     -0.45     -0.30     -0.15

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00258

Critical value = 4.29

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6       7

2  -0.2925

0.2174

3  -0.1976     -0.1647

0.3874      0.4296

4  -0.2371     -0.2050     -0.3743

0.2727      0.3156      0.2201

5  -0.2978     -0.2656     -0.4350     -0.3210

0.2120      0.2550      0.1594      0.1996

6  -0.2785     -0.2466     -0.4178     -0.3020     -0.2413

0.2025      0.2458      0.1521      0.1904      0.2511

7  -0.2428     -0.2111     -0.3834     -0.2665     -0.2058     -0.1952

0.2214      0.2648      0.1722      0.2095      0.2701      0.2498

8  -0.3551     -0.3232     -0.4938     -0.3785     -0.3178     -0.3080  -0.3267

0.1342      0.1774      0.0831      0.1221      0.1827      0.1632   0.1272

Tukey’s test – TP – Winter - 1999

Source     DF        SS        MS        F        P

W           7     0.561     0.080     0.70    0.670

Error     340    38.748     0.114

Total     347    39.308

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  ---+---------+---------+---------+---

1          50   -1.0910    0.3116                 (--------*--------)

2          38   -1.2183    0.4920  (----------*----------)

3          31   -1.1500    0.2242        (-----------*-----------)

4          74   -1.1744    0.2284          (-------*------)

5          44   -1.1224    0.3340             (---------*---------)

6          52   -1.1958    0.2663      (--------*---------)

7          37   -1.1285    0.5474           (----------*----------)

8          22   -1.1278    0.1573        (-------------*-------------)

---+---------+---------+---------+---

Pooled StDev =   0.3376            -1.30     -1.20     -1.10     -1.00

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00260

Critical value = 4.29

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6        7

2  -0.0931

0.3477

3  -0.1751     -0.3162

0.2931      0.1795

4  -0.1040     -0.2482     -0.1946

0.2709      0.1605      0.2436

5  -0.1802     -0.3226     -0.2677     -0.2470

0.2431      0.1309      0.2126      0.1429

6  -0.0980     -0.2411     -0.1866     -0.1640     -0.1364

0.3076      0.1960      0.2782      0.2067      0.2831

7  -0.1846     -0.3263     -0.2708     -0.2521     -0.2224     -0.2875

0.2596      0.1467      0.2279      0.1602      0.2345      0.1530

8  -0.2252     -0.3649     -0.3077     -0.2953     -0.2620     -0.3285  -0.2764

0.2988      0.1838      0.2633      0.2020      0.2728      0.1925   0.2750

Tukey’s test – TKN – Winter - 1999

Source     DF        SS        MS        F        P

W           7    0.4373    0.0625     0.75    0.627

Error     340   28.1854    0.0829

Total     347   28.6227

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  --------+---------+---------+--------

1          50    0.4951    0.1350            (-------*-------)

2          38    0.4140    0.3825  (--------*---------)

3          31    0.5332    0.2055             (---------*---------)

4          74    0.5106    0.2907              (------*------)

5          44    0.5228    0.2246              (-------*--------)

6          52    0.5303    0.2671               (-------*-------)

7          37    0.4691    0.4813        (--------*--------)

8          22    0.4809    0.1190      (-----------*-----------)

--------+---------+---------+--------

Pooled StDev =   0.2879                  0.40      0.50      0.60

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00260

Critical value = 4.29

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6        7

2  -0.1068

0.2691

3  -0.2377     -0.3306

0.1616      0.0922

4  -0.1753     -0.2709     -0.1643

0.1444      0.0777      0.2095

5  -0.2082     -0.3022     -0.1944     -0.1785

0.1529      0.0846      0.2152      0.1541

6  -0.2081     -0.3027     -0.1953     -0.1777     -0.1864

0.1379      0.0701      0.2011      0.1384      0.1714

7  -0.1633     -0.2568     -0.1485     -0.1343     -0.1411     -0.1267

0.2155      0.1467      0.2768      0.2174      0.2485      0.2490

8  -0.2092     -0.3009     -0.1912     -0.1824     -0.1861     -0.1728  -0.2470

0.2377      0.1671      0.2958      0.2418      0.2700      0.2715   0.2233

Tukey’s test – NOx – Winter - 1999

Source     DF        SS        MS        F        P

W           7     4.001     0.572     5.10    0.000

Error     340    38.136     0.112

Total     347    42.137

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  ---------+---------+---------+-------

1          50   -1.6297    0.6012                          (-----*------)

2          38   -1.8272    0.2905            (------*------)

3          31   -1.9612    0.1287  (-------*-------)

4          74   -1.8702    0.2650           (----*----)

5          44   -1.7024    0.3174                     (------*-----)

6          52   -1.8538    0.2289           (-----*------)

7          37   -1.9170    0.1534      (------*------)

8          22   -1.6828    0.4269                   (---------*--------)

---------+---------+---------+-------

Pooled StDev =   0.3349                  -1.95     -1.80     -1.65

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00260

Critical value = 4.29

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6        7

2  -0.0211

0.4162

3   0.0992     -0.1119

0.5637      0.3798

4   0.0545     -0.1598     -0.3083

0.4265      0.2457      0.1264

5  -0.1373     -0.3498     -0.4970     -0.3612

0.2827      0.1002     -0.0205      0.0256

6   0.0229     -0.1902     -0.3379     -0.2002     -0.0567

0.4253      0.2434      0.1232      0.1675      0.3595

7   0.0670     -0.1449     -0.2916     -0.1578     -0.0120     -0.1553

0.5076      0.3244      0.2032      0.2513      0.4412      0.2817

8  -0.2068     -0.4165     -0.5615     -0.4341     -0.2848     -0.4294  -0.5077

0.3131      0.1278      0.0049      0.0593      0.2457      0.0874   0.0394

Tukey’s test – NH3 – Winter – 1999

Source     DF        SS        MS        F        P

W           7    1.6877    0.2411     5.14    0.000

Error     340   15.9624    0.0469

Total     347   17.6501

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  --------+---------+---------+--------

1          50   -0.8703    0.3043  (----*-----)

2          38   -0.7827    0.2125         (-----*-----)

3          31   -0.7563    0.2536           (-----*-----)

4          74   -0.7273    0.2175               (---*----)

5          44   -0.6336    0.1340                      (----*-----)

6          52   -0.7538    0.1874            (----*----)

2          38   -0.7827    0.2125         (-----*-----) r M

8          22   -0.6339    0.1394                    (------*-------)

--------+---------+---------+--------

Pooled StDev =   0.2167                 -0.84     -0.72     -0.60

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00260

Critical value = 4.29

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6         7

2  -0.2290

0.0539

3  -0.2642     -0.1855

0.0363      0.1326

4  -0.2633     -0.1866     -0.1696

-0.0226      0.0758      0.1116

5  -0.3725     -0.2947     -0.2768     -0.2189

-0.1008     -0.0036      0.0314      0.0314

6  -0.2466     -0.1691     -0.1516     -0.0924     -0.0144

0.0138      0.1114      0.1467      0.1455      0.2549

7  -0.2707     -0.1924     -0.1742     -0.1175     -0.0381     -0.1531

0.0144      0.1112      0.1459      0.1471      0.2551      0.1296

8  -0.4045     -0.3249     -0.3056     -0.2530     -0.1713     -0.2871  -0.2852

-0.0682      0.0273      0.0608      0.0662      0.1719      0.0472   0.0687

Tukey’s test – NH3 – Summer – 2000

Source     DF        SS        MS        F        P

S           7     1.818     0.260     1.14    0.350

Error      52    11.797     0.227

Total      59    13.616

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  -+---------+---------+---------+-----

1           8   -0.8803    0.5185                  (---------*---------)

2           7   -1.3169    0.3951     (---------*----------)

3           5   -1.3442    0.4011  (------------*-----------)

4           8   -1.0078    0.5259               (--------*---------)

5           8   -0.9350    0.4057                 (--------*---------)

6           5   -1.1830    0.3564       (-----------*-----------)

7          11   -0.9720    0.4543                 (-------*-------)

8           8   -0.8242    0.6182                    (--------*---------)

-+---------+---------+---------+-----

Pooled StDev =   0.4763          -1.75     -1.40     -1.05     -0.70

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00262

Critical value = 4.47

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6       7

2  -0.3425

1.2158

3  -0.3943     -0.8542

1.3222      0.9089

4  -0.6253     -1.0883     -1.1948

0.8802      0.4700      0.5218

5  -0.6980     -1.1611     -1.2675     -0.8255

0.8075      0.3973      0.4490      0.6800

6  -0.5555     -1.0154     -1.1134     -0.6830     -0.6103

1.1610      0.7476      0.7909      1.0335      1.1063

7  -0.6078     -1.0728     -1.1842     -0.7353     -0.6625     -1.0230

0.7913      0.3830      0.4398      0.6638      0.7366      0.6010

8  -0.8089     -1.2719     -1.3784     -0.9364     -0.8636     -1.2171  -0.8474

0.6966      0.2864      0.3382      0.5691      0.6419      0.4994   0.5517

Tukey’s test – NOx – Summer – 2000

Source     DF        SS        MS        F        P

S           7    0.1797    0.0257     0.56    0.788

Error      52    2.4011    0.0462

Total      59    2.5808

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  -------+---------+---------+---------

1           8   -2.0000    0.0000      (---------*---------)

2           7   -2.0000    0.0000     (----------*----------)

3           5   -2.0000    0.0000   (------------*------------)

4           8   -2.0000    0.0000      (---------*---------)

5           8   -2.0000    0.0000      (---------*---------)

6           5   -2.0000    0.0000   (------------*------------)

7          11   -1.8585    0.4692                (--------*--------)

8           8   -1.9404    0.1687         (----------*---------)

-------+---------+---------+---------

Pooled StDev =   0.2149                -2.10     -1.95     -1.80

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00262

Critical value = 4.47

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6        7

2  -0.3515

0.3515

3  -0.3872     -0.3977

0.3872      0.3977

4  -0.3396     -0.3515     -0.3872

0.3396      0.3515      0.3872

5  -0.3396     -0.3515     -0.3872     -0.3396

0.3396      0.3515      0.3872      0.3396

6  -0.3872     -0.3977     -0.4296     -0.3872     -0.3872

0.3872      0.3977      0.4296      0.3872      0.3872

7  -0.4571     -0.4699     -0.5078     -0.4571     -0.4571     -0.5078

0.1741      0.1869      0.2248      0.1741      0.1741      0.2248

8  -0.3992     -0.4112     -0.4468     -0.3992     -0.3992     -0.4468  -0.2338

0.2800      0.2919      0.3276      0.2800      0.2800      0.3276   0.3974

Tukey’s test – TKN – Summer – 2000

Source     DF        SS        MS        F        P

S          7    1.2179    0.1740     4.29    0.001

Error      52    2.1081    0.0405

Total      59    3.3260

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  ----+---------+---------+---------+--

1           8    0.0536    0.2229  (-----*-----)

2           7    0.2340    0.1247         (-----*-----)

3           5    0.2723    0.1101          (------*------)

4           8    0.3252    0.0988             (-----*-----)

5           8    0.3975    0.1449                (-----*-----)

6           5    0.4623    0.2539                 (------*-------)

7          11    0.2992    0.2561             (----*----)

8           8    0.5525    0.2645                      (-----*-----)

----+---------+---------+---------+--

Pooled StDev =   0.2013              0.00      0.25      0.50      0.75

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00262

Critical value = 4.47

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6        7

2  -0.5098

0.1489

3  -0.5815     -0.4109

0.1441      0.3344

4  -0.5899     -0.4206     -0.4158

0.0465      0.2382      0.3099

5  -0.6622     -0.4929     -0.4881     -0.3905

-0.0258      0.1659      0.2375      0.2459

6  -0.7716     -0.6009     -0.5925     -0.4999     -0.4276

-0.0459      0.1444      0.2125      0.2257      0.2980

7  -0.5414     -0.3729     -0.3702     -0.2697     -0.1974     -0.1802

0.0501      0.2425      0.3163      0.3217      0.3941      0.5064

8  -0.8172     -0.6479     -0.6430     -0.5455     -0.4732     -0.4530  -0.5490

-0.1807      0.0109      0.0826      0.0909      0.1632      0.2726   0.0424

Tukey’s test – TP – Summer – 2000

Source     DF        SS        MS        F        P

S           7     5.456     0.779     4.94    0.000

Error      52     8.205     0.158

Total      59    13.661

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  ---+---------+---------+---------+---

1           8   -1.3358    0.3469   (----*-----)

2           7   -0.9032    0.3575           (-----*-----)

3           5   -0.9738    0.3646        (-------*------)

4           8   -0.5956    0.4026                 (-----*-----)

5           8   -0.4842    0.3093                    (----*-----)

6           5   -0.5459    0.4502                 (------*------)

7          11   -0.7598    0.4032               (----*----)

8           8   -0.3409    0.5103                       (----*-----)

---+---------+---------+---------+---

Pooled StDev =   0.3972            -1.50     -1.00     -0.50      0.00

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00262

Critical value = 4.47

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6        7

2  -1.0824

0.2172

3  -1.0778     -0.6646

0.3538      0.8057

4  -1.3679     -0.9574     -1.0940

-0.1124      0.3422      0.3376

5  -1.4794     -1.0688     -1.2054     -0.7392

-0.2238      0.2308      0.2262      0.5164

6  -1.5057     -1.0925     -1.2220     -0.7655     -0.6541

-0.0741      0.3778      0.3662      0.6660      0.7775

7  -1.1594     -0.7505     -0.8912     -0.4192     -0.3078     -0.4633

0.0074      0.4636      0.4632      0.7476      0.8590      0.8911

8  -1.6226     -1.2121     -1.3486     -0.8824     -0.7710     -0.9207  -1.0022

-0.3671      0.0875      0.0829      0.3731      0.4845      0.5108   0.1646

Tukey’s test – TP– Winter – 2000

Source     DF        SS        MS        F        P

W           7    3.2243    0.4606     5.39    0.001

Error      27    2.3093    0.0855

Total      34    5.5335

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  ---------+---------+---------+-------

1           5   -0.8834    0.4354        (----*-----)

2           6   -0.8977    0.1735        (----*----)

3           4   -0.3650    0.2809                  (-----*-----)

4           5   -0.9268    0.1465       (----*-----)

5           4   -0.5900    0.2645             (-----*-----)

6           4   -0.1343    0.3047                      (-----*-----)

7           3   -0.6259    0.0709            (-----*------)

8           4   -1.1435    0.4395  (-----*-----)

---------+---------+---------+-------

Pooled StDev =   0.2925                  -1.00     -0.50      0.00

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00285

Critical value = 4.64

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6       7

2  -0.5668

0.5953

3  -1.1620     -1.1520

0.1253      0.0868

4  -0.5635     -0.5519     -0.0819

0.6503      0.6102      1.2054

5  -0.9371     -0.9270     -0.4535     -0.9805

0.3503      0.3117      0.9035      0.3069

6  -1.3928     -1.3827     -0.9092     -1.4362     -1.1342

-0.1054     -0.1440      0.4478     -0.1488      0.2228

7  -0.9582     -0.9502     -0.4720     -1.0016     -0.6969     -0.2412

0.4433      0.4068      0.9937      0.3999      0.7688      1.2245

8  -0.3836     -0.3736      0.0999     -0.4270     -0.1250      0.3307  -0.2153

0.9037      0.8652      1.4569      0.8603      1.2320      1.6876   1.2504

Tukey’s test – TKN– Winter – 2000

Source     DF        SS        MS        F        P

W           7    1.0621    0.1517     1.92    0.105

Error      27    2.1302    0.0789

Total      34    3.1923

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  ------+---------+---------+---------+

1           5    0.5648    0.1685                 (------*-------)

2           6    0.6067    0.2391                   (-----*------)

3           4    0.6971    0.5596                    (-------*-------)

4           5    0.4773    0.1679              (-------*------)

5           4    0.0914    0.4327  (--------*-------)

6           4    0.6940    0.1897                    (-------*-------)

7           3    0.5872    0.0129               (---------*--------)

8           4    0.5829    0.0532                (--------*-------)

------+---------+---------+---------+

Pooled StDev =   0.2809                0.00      0.35      0.70      1.05

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00285

Critical value = 4.64

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6        7

2  -0.6000

0.5161

3  -0.7505     -0.6852

0.4859      0.5045

4  -0.4954     -0.4286     -0.3984

0.6703      0.6875      0.8380

5  -0.1448     -0.0795     -0.0460     -0.2323

1.0916      1.1102      1.2573      1.0041

6  -0.7475     -0.6822     -0.6486     -0.8350     -1.2543

0.4889      0.5076      0.6547      0.4015      0.0490

7  -0.6955     -0.6321     -0.5940     -0.7829     -1.1997     -0.5970

0.6506      0.6712      0.8137      0.5631      0.2080      0.8107

8  -0.6364     -0.5710     -0.5375     -0.7238     -1.1432     -0.5405  -0.6996

0.6001      0.6187      0.7658      0.5126      0.1601      0.7628   0.7082

Tukey’s test – NOx– Winter – 2000

Source     DF        SS        MS        F        P

W           7     3.258     0.465     0.96    0.478

Error      27    13.070     0.484

Total      34    16.327

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  ---------+---------+---------+-------

1           5   -1.7772    0.4982      (---------*--------)

2           6   -1.0943    1.0354                 (-------*--------)

3           4   -2.0000    0.0000  (---------*----------)

4           5   -1.2660    0.7238              (--------*--------)

5           4   -1.5689    0.8621        (----------*---------)

6           4   -1.8495    0.3010    (----------*---------)

7           3   -1.3010    0.0000           (----------*-----------)

8           4   -1.5797    0.8406        (---------*----------)

---------+---------+---------+-------

Pooled StDev =   0.6957                  -2.10     -1.40     -0.70

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00285

Critical value = 4.64

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6       7

2  -2.0652

0.6994

3  -1.3085     -0.5678

1.7541      2.3792

4  -1.9550     -1.2106     -2.2653

0.9325      1.5539      0.7972

5  -1.7396     -0.9989     -2.0452     -1.2283

1.3230      1.9481      1.1831      1.8343

6  -1.4590     -0.7183     -1.7646     -0.9478     -1.3336

1.6036      2.2287      1.4636      2.1148      1.8947

7  -2.1432     -1.4074     -2.4424     -1.6320     -2.0114     -2.2919

1.1909      1.8208      1.0445      1.7021      1.4756      1.1950

8  -1.7288     -0.9881     -2.0344     -1.2176     -1.6034     -1.8839  -1.4648

1.3338      1.9589      1.1938      1.8450      1.6249      1.3443   2.0221

Tukey’s test – NH3– Winter – 2000

Source     DF        SS        MS        F        P

W           7     1.693     0.242     0.94    0.496

Error      27     6.980     0.259

Total      34     8.673

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  ----------+---------+---------+------

1           5   -0.4120    0.7171    (---------*--------)

2           6   -0.2207    0.5776         (--------*-------)

3           4   -0.3637    0.6385    (----------*---------)

4           5   -0.4960    0.4846   (--------*--------)

5           4    0.2615    0.1215                 (---------*----------)

6           4   -0.4230    0.5200   (----------*---------)

7           3   -0.3732    0.1434  (------------*-----------)

8           4   -0.4099    0.2554   (----------*---------)

----------+---------+---------+------

Pooled StDev =   0.5084                   -0.50      0.00      0.50

Tukey's pairwise comparisons

Family error rate = 0.0500

Individual error rate = 0.00285

Critical value = 4.64

Intervals for (column level mean) - (row level mean)

1           2           3           4           5           6       7

2  -1.2013

0.8189

3  -1.1673     -0.9338

1.0708      1.2198

4  -0.9710     -0.7349     -0.9868

1.1391      1.2854      1.2513

5  -1.7925     -1.5590     -1.8047     -1.8765

0.4456      0.5946      0.5544      0.3616

6  -1.1080     -0.8745     -1.1203     -1.1920     -0.4951

1.1301      1.2791      1.2389      1.0461      1.8640

7  -1.2571     -1.0272     -1.2646     -1.3411     -0.6395     -1.3239

1.1795      1.3320      1.2835      1.0954      1.9087      1.2242

8  -1.1211     -0.8876     -1.1334     -1.2051     -0.5082     -1.1927  -1.2373

1.1170      1.2660      1.2258      1.0330      1.8510      1.1665   1.3109

Tukey’s test – NH3– Summer – 2001

Source     DF        SS        MS        F        P

NH3         7     4.263     0.609     5.63    0.000

Error     765    82.682     0.108

Total     772    86.945

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  -------+---------+---------+---------

S1_NH4_l   98   -0.3757    0.4604                         (-----*------)

S8_NH4_l  103   -0.6002    0.2597   (-----*-----)

S4_NH4_l   97   -0.5882    0.2650    (-----*------)

S6_NH4_l   94   -0.4794    0.2835              (------*------)

S2_NH4_l   92   -0.4689    0.2820               (------*------)

S7_NH4_l  108   -0.5650    0.3331      (-----*------)

S3_NH4_l   89   -0.5526    0.2570       (------*------)

S5_NH4_l   92   -0.4498    0.4207                 (------*------)

-------+---------+---------+---------

Pooled StDev =   0.3288                -0.60     -0.50     -0.40

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NH3 - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | 0.0838 |  |  |  |  |  |  |
|  |  | 0.3652 |  |  |  |  |  |  |
| Low | S4 | 0.0697 | -0.1292 |  |  |  |  |  |
|  |  | 0.3554 | 0.153 |  |  |  |  |  |
|  | S6 | -0.0403 | -0.0215 | -0.2532 |  |  |  |  |
|  |  | 0.2477 | 0.263 | 0.0355 |  |  |  |  |
| Medium | S2 | -0.0515 | -0.0118 | -0.0258 | -0.1358 |  |  |  |
|  |  | 0.238 | 0.2743 | 0.2644 | 0.1567 |  |  |  |
|  | S7 | 0.0502 | -0.1022 | -0.1627 | -0.055 | -0.0454 |  |  |
|  |  | 0.3285 | 0.1725 | 0.1163 | 0.2263 | 0.2376 |  |  |
| High | S3 | 0.0309 | -0.0968 | -0.1108 | -0.2207 | -0.0646 | -0.1303 |  |
|  |  | 0.3229 | 0.1919 | 0.182 | 0.0743 | 0.2319 | 0.1552 |  |
|  | S5 | -0.0707 | 0.0073 | -0.2836 | -0.1166 | -0.1662 | -0.0262 | -0.2511 |
|  |  | 0.2188 | 0.2935 | 0.0067 | 0.1759 | 0.1279 | 0.2568 | 0.0454 |

Conclusion: it is impossible to distinguish each other most of the means. The positive results we obtain for the Control treatment seems only reflect an abnormal mean for S1.

#### Tukey’s test – NH3– Winter – 2001

Source     DF        SS        MS        F        P

NH3         7    9.0144    1.2878    14.02    0.000

Error     880   80.8104    0.0918

Total     887   89.8248

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  ----------+---------+---------+------

W4        119   -0.6029    0.3834                  (---*--)

W7        114   -0.7375    0.2438         (---*---)

W1        113   -0.6730    0.2141             (---*---)

W6        101   -0.6944    0.2241            (---*---)

W2        115   -0.4835    0.3594                          (---*--)

W8        111   -0.7877    0.2930      (--*---)

W3         91   -0.8319    0.3338  (----*---)

W5        124   -0.7088    0.3175           (---*--)

----------+---------+---------+------

Pooled StDev =   0.3030                   -0.75     -0.60     -0.45

| Tukey's Procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NH3 - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | 0.0141 |  |  |  |  |  |  |
|  |  | 0.255 |  |  |  |  |  |  |
| Low | W1 | -0.1908 | -0.0575 |  |  |  |  |  |
|  |  | 0.0507 | 0.1865 |  |  |  |  |  |
|  | W6 | -0.0329 | -0.0823 | -0.1044 |  |  |  |  |
|  |  | 0.2159 | 0.1687 | 0.1473 |  |  |  |  |
| Medium | W2 | -0.0008 | 0.1325 | -0.3112 | 0.0856 |  |  |  |
|  |  | 0.2396 | 0.3755 | -0.0677 | 0.3363 |  |  |  |
|  | W8 | 0.0635 | -0.0724 | -0.0081 | -0.0331 | 0.1819 |  |  |
|  |  | 0.3061 | 0.1728 | 0.2376 | 0.2197 | 0.4265 |  |  |
| High | W3 | -0.357 | -0.2236 | 0.0294 | -0.2703 | 0.2194 | -0.1742 |  |
|  |  | -0.101 | 0.0348 | 0.2884 | -0.0046 | 0.4774 | 0.0858 |  |
|  | W5 | -0.0121 | -0.0906 | -0.0837 | -0.1376 | 0.1063 | -0.0412 | -0.25 |
|  |  | 0.2239 | 0.1479 | 0.1554 | 0.1088 | 0.3443 | 0.199 | 0.0038 |

Conclusion: it is again very difficult to see a difference between the means. We cannot trust the few positive results we find because of the negative results for the replicates plots.

#### Tukey’s test – NOx– Summer – 2001

Source     DF        SS        MS        F        P

NO x        7     2.639     0.377     1.74    0.098

Error     766   166.406     0.217

Total     773   169.045

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  --+---------+---------+---------+----

S1         98   -1.6502    0.6042                   (------*-------)

S8        103   -1.8271    0.3611    (-------*------)

S4         97   -1.8201    0.4212     (------*-------)

S6         94   -1.7890    0.3820       (-------*-------)

S2         92   -1.7663    0.4568         (-------*-------)

S7        109   -1.7302    0.5402             (------*------)

S3         89   -1.7633    0.5185         (-------*-------)

S5         92   -1.8397    0.3769   (-------*-------)

--+---------+---------+---------+----

Pooled StDev =   0.4661           -1.92     -1.80     -1.68     -1.56

| Tukey's procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOx -Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | -0.0226 |  |  |  |  |  |  |
|  |  | 0.3764 |  |  |  |  |  |  |
| Low | S4 | -0.0327 | -0.193 |  |  |  |  |  |
|  |  | 0.3723 | 0.2071 |  |  |  |  |  |
|  | S6 | -0.0653 | -0.1636 | -0.2357 |  |  |  |  |
|  |  | 0.3429 | 0.2398 | 0.1736 |  |  |  |  |
| Medium | S2 | -0.891 | -0.142 | -0.152 | -0.1847 |  |  |  |
|  |  | 0.3214 | 0.2636 | 0.2595 | 0.23 |  |  |  |
|  | S7 | -0.1168 | -0.0974 | -0.2872 | -0.2578 | -0.2363 |  |  |
|  |  | 0.2768 | 0.2912 | 0.1075 | 0.1402 | 0.1641 |  |  |
| High | S3 | -0.0939 | -0.1408 | -0.1508 | -0.1834 | -0.2131 | -0.2351 |  |
|  |  | 0.3201 | 0.2684 | 0.2643 | 0.2348 | 0.2072 | 0.1689 |  |
|  | S5 | -0.157 | -0.2154 | -0.1861 | -0.2581 | -0.1351 | -0.3097 | -0.1338 |
|  |  | 0.3948 | 0.1902 | 0.2254 | 0.1566 | 0.2819 | 0.0907 | 0.2866 |

Conclusion: all the data are here confined in the same range. No conclusion is allowed.

#### Tukey’s test – NOx– Winter – 2001

Source     DF        SS        MS        F        P

NO x        7    37.358     5.337    14.41    0.000

Error     879   325.598     0.370

Total     886   362.956

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  --------+---------+---------+--------

W4        118   -1.4341    0.6279                  (----*---)

W7        114   -1.6772    0.4799        (----*---)

W1        113   -1.2644    0.7829                         (---*----)

W6        101   -1.6547    0.5167         (----*----)

W2        115   -1.2443    0.8519                          (---*----)

W8        111   -1.8156    0.3471   (---*----)

W3         91   -1.7687    0.4015    (----*----)

W5        124   -1.4587    0.6222                 (----*---)

--------+---------+---------+--------

Pooled StDev =   0.6086                 -1.75     -1.50     -1.25

| Tukey's Procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOx - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | 0.0006 |  |  |  |  |  |  |
|  |  | 0.4856 |  |  |  |  |  |  |
| Low | W1 | -0.0732 | 0.1678 |  |  |  |  |  |
|  |  | 0.4128 | 0.658 |  |  |  |  |  |
|  | W6 | -0.0297 | -0.2297 | 0.1375 |  |  |  |  |
|  |  | 0.4708 | 0.2748 | 0.6431 |  |  |  |  |
| Medium | W2 | -0.0521 | 0.1889 | -0.2646 | 0.1586 |  |  |  |
|  |  | 0.4318 | 0.6769 | 0.2245 | 0.6622 |  |  |  |
|  | W8 | 0.1374 | -0.1078 | 0.3046 | -0.0929 | 0.3257 |  |  |
|  |  | 0.6256 | 0.3846 | 0.798 | 0.4148 | 0.817 |  |  |
| High | W3 | -0.5921 | -0.351 | 0.2443 | -0.3809 | 0.2654 | -0.2142 |  |
|  |  | -0.077 | 0.1681 | 0.7644 | 0.1528 | 0.7834 | 0.308 |  |
|  | W5 | -0.2128 | -0.0211 | -0.0457 | -0.0515 | -0.0246 | 0.1156 | -0.5648 |
|  |  | 0.262 | 0.4581 | 0.4345 | 0.4434 | 0.4535 | 0.5981 | -0.0551 |

Conclusion: the bad values for the replicate tests avoid drawing any conclusion.

#### Tukey’s test – TKN– Summer – 2001

Source     DF        SS        MS        F        P

TKN         7     1.757     0.251     1.92    0.063

Error     764    99.836     0.131

Total     771   101.593

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  -------+---------+---------+---------

S1         98    0.5428    0.3127                (------*------)

S8        102    0.4571    0.4865        (------*------)

S4         97    0.4670    0.2666        (-------*------)

S6         94    0.4826    0.4201          (------*-------)

S2         92    0.5708    0.3137                   (------*------)

S7        108    0.4642    0.2996         (-----*------)

S3         89    0.5002    0.2636           (-------*-------)

S5         92    0.4060    0.4515  (-------*------)

-------+---------+---------+---------

Pooled StDev =   0.3615                 0.40      0.50      0.60

| Tukey's Procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TKN - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | -0.0694 |  |  |  |  |  |  |
|  |  | 0.2408 |  |  |  |  |  |  |
| Low | S4 | -0.0812 | -0.1456 |  |  |  |  |  |
|  |  | 0.2329 | 0.1654 |  |  |  |  |  |
|  | S6 | -0.0982 | -0.1312 | -0.1744 |  |  |  |  |
|  |  | 0.2185 | 0.1824 | 0.1431 |  |  |  |  |
| Medium | S2 | -0.1872 | -0.0439 | -0.0558 | -0.0726 |  |  |  |
|  |  | 0.1312 | 0.2714 | 0.2634 | 0.249 |  |  |  |
|  | S7 | -0.0744 | -0.1442 | -0.1506 | -0.1361 | -0.049 |  |  |
|  |  | 0.2315 | 0.1586 | 0.1561 | 0.1731 | 0.2622 |  |  |
| High | S3 | -0.1179 | -0.116 | -0.1278 | -0.1447 | -0.0924 | -0.1211 |  |
|  |  | 0.2032 | 0.2022 | 0.1941 | -0.1797 | 0.2337 | 0.1929 |  |
|  | S5 | -0.0224 | -0.2088 | -0.0986 | -0.2375 | 0.0031 | -0.2138 | -0.0688 |
|  |  | 0.296 | 0.1066 | 0.2206 | 0.0842 | 0.3265 | 0.0973 | 0.2572 |

Conclusion: most of the data are confined in the same range. The 2 positive results are not enough to draw viable conclusions.

#### Tukey’s test – TKN– Winter – 2001

Source     DF        SS        MS        F        P

TKN         7    1.6613    0.2373     3.97    0.000

Error     880   52.6131    0.0598

Total     887   54.2744

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  ---------+---------+---------+-------

W4        119    0.4304    0.2334                (-----*------)

W7        114    0.4412    0.2312                  (-----*-----)

W1        113    0.3749    0.2718        (------*-----)

W6        101    0.4000    0.2840           (------*------)

W2        115    0.4938    0.2014                         (------*-----)

W8        111    0.3395    0.3184   (-----*------)

W3         91    0.4121    0.1894             (------*------)

W5        124    0.4079    0.1974             (-----*-----)

---------+---------+---------+-------

Pooled StDev =   0.2445                   0.350     0.420     0.490

| Tukey's Procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TKN - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | -0.108 |  |  |  |  |  |  |
|  |  | 0.0864 |  |  |  |  |  |  |
| Low | W1 | -0.1529 | -0.1648 |  |  |  |  |  |
|  |  | 0.0419 | 0.0322 |  |  |  |  |  |
|  | W6 | -0.07 | -0.1425 | -0.1267 |  |  |  |  |
|  |  | 0.1307 | 0.0602 | 0.0764 |  |  |  |  |
| Medium | W2 | -0.0336 | -0.0454 | -0.2171 | -0.0074 |  |  |  |
|  |  | 0.1604 | 0.1506 | -0.0206 | 0.1949 |  |  |  |
|  | W8 | -0.007 | 0.0028 | -0.0637 | -0.0415 | 0.0556 |  |  |
|  |  | 0.1888 | 0.2006 | 0.1345 | 0.1625 | 0.253 |  |  |
| High | W3 | -0.1216 | -0.1334 | -0.1417 | -0.0951 | -0.0224 | -0.0323 |  |
|  |  | 0.085 | 0.0752 | 0.0673 | 0.1193 | 0.1858 | 0.1775 |  |
|  | W5 | -0.0728 | -0.1295 | -0.1295 | -0.0915 | -0.0102 | -0.0284 | -0.0983 |
|  |  | 0.1176 | 0.063 | 0.0634 | 0.1074 | 0.1818 | 0.1654 | 0.1065 |

Conclusion: no constructive conclusion can be drawn from this test.

#### Tukey’s test – TP– Summer – 2001

Source     DF        SS        MS        F        P

TP          7     6.225     0.889     3.47    0.001

Error     765   196.065     0.256

Total     772   202.290

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  ----+---------+---------+---------+--

S1         98   -0.2036    0.5240                (-----*------)

S8        102   -0.2828    0.5681           (-----*------)

S4         97   -0.1382    0.4392                    (------*------)

S6         94   -0.2915    0.5335          (------*-----)

S2         92   -0.1763    0.3936                 (------*------)

S7        109   -0.3933    0.5052   (------*-----)

S3         89   -0.2458    0.4046             (------*------)

S5         92   -0.3991    0.6270  (------*------)

----+---------+---------+---------+--

Pooled StDev =   0.5063             -0.45     -0.30     -0.15      0.00

| Tukey's Procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TP - Summer |  | S1 | S8 | S4 | S6 | S2 | S7 | S3 |
| Control | S8 | -0.138 |  |  |  |  |  |  |
|  |  | 0.2965 |  |  |  |  |  |  |
| Low | S4 | -0.2853 | -0.0732 |  |  |  |  |  |
|  |  | 0.1546 | 0.3624 |  |  |  |  |  |
|  | S6 | -0.1338 | -0.2282 | -0.069 |  |  |  |  |
|  |  | 0.3096 | 0.2109 | 0.3756 |  |  |  |  |
| Medium | S2 | -0.2503 | -0.1142 | -0.2615 | -0.11 |  |  |  |
|  |  | 0.1956 | 0.3274 | 0.1855 | 0.3405 |  |  |  |
|  | S7 | -0.0241 | -0.322 | 0.0407 | -0.1144 | -0.0004 |  |  |
|  |  | 0.4035 | 0.1011 | 0.4694 | 0.3179 | 0.4345 |  |  |
| High | S3 | -0.1826 | -0.1858 | -0.333 | -0.1815 | -0.1587 | -0.0719 |  |
|  |  | 0.2671 | 0.2598 | 0.1178 | 0.2728 | 0.2979 | 0.3669 |  |
|  | S5 | -0.0274 | -0.3371 | 0.0374 | -0.3328 | -0.0036 | -0.2232 | -0.075 |
|  |  | 0.4184 | 0.1045 | 0.4844 | 0.1176 | 0.4493 | 0.2116 | 0.3816 |

Conclusion: The distributions are too superposed to be capable providing a conclusion.

#### Tukey’s test – TP – Summer – 2001

Source     DF        SS        MS        F        P

TP          7    12.803     1.829     9.74    0.000

Error     880   165.275     0.188

Total     887   178.078

Individual 95% CIs For Mean

Based on Pooled StDev

Level       N      Mean     StDev  ----+---------+---------+---------+--

W4        119   -1.1719    0.4767        (----*----)

W7        114   -0.9879    0.3619                   (----*----)

W1        113   -1.0774    0.4195              (----*----)

W6        101   -0.9046    0.4028                        (----*-----)

W2        115   -1.0334    0.4218                (----*----)

W8        111   -1.2588    0.4194  (----*----)

W3         91   -0.8655    0.5602                          (-----*----)

W5        124   -0.9890    0.4018                   (----*----)

----+---------+---------+---------+--

Pooled StDev =   0.4334             -1.28     -1.12     -0.96     -0.80

| Tukey's Procedure |  | Control |  | Low |  | Medium |  | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TP - Winter |  | W4 | W7 | W1 | W6 | W2 | W8 | W3 |
| Control | W7 | -0.3563 |  |  |  |  |  |  |
|  |  | -0.0118 |  |  |  |  |  |  |
| Low | W1 | -0.0782 | -0.2641 |  |  |  |  |  |
|  |  | 0.2672 | 0.085 |  |  |  |  |  |
|  | W6 | -0.4452 | -0.0963 | -0.3529 |  |  |  |  |
|  |  | -0.0895 | 0.263 | 0.0071 |  |  |  |  |
| Medium | W2 | -0.0334 | -0.2193 | -0.2182 | -0.3081 |  |  |  |
|  |  | 0.3104 | 0.1282 | 0.1301 | 0.0505 |  |  |  |
|  | W8 | -0.0865 | 0.0957 | 0.0057 | 0.1735 | 0.0505 |  |  |
|  |  | 0.2604 | 0.4463 | 0.3571 | 0.5351 | 0.4004 |  |  |
| High | W3 | 0.1233 | -0.0624 | -0.3971 | -0.151 | -0.3523 | 0.2074 |  |
|  |  | 0.4895 | 0.3072 | -0.0267 | 0.2291 | 0.0166 | 0.5792 |  |
|  | W5 | -0.3516 | -0.1717 | -0.2594 | -0.2606 | -0.2146 | 0.0981 | -0.058 |
|  |  | -0.0142 | 0.1695 | 0.0825 | 0.0918 | 0.1258 | 0.4417 | 0.3049 |

Conclusion: we have here more positive results but it is difficult to trust them because of the replicate tests. Anyway, we cannot identify a general law to identify the different means.